Oak RGB
RGB Color Sensor

Datasheet

Originally authored by Toradex AG. This work is now available under the terms and conditions of the Creative Commons License 'Attribution CC BY'

Details of which can be found here:  http://creativecommons.org/licenses/by/3.0/

Contents

1.	Introduction	3

1.1	Reference Documents	3

2.	Hardware Specifications	4

2.1	Sensor: Avago ADJD-S313-QR999	4

2.2	Measurement Range	4

2.3	Supported Sensor Features	4

2.4	USB Interface	4

3.	Software Specifications	5

3.1	INTERRUPT IN Report Contents (Real time data)	5

3.2	FEATURE Report Commands	5

4.	Technical Specifications	9

4.1	Electrical Specifications	9

4.2	Mechanical Dimensions	9

4.3	RoHS Compliance	9

# Introduction

…

The Oak RGB can be integrated in a custom application very easily. The operating power as well as real time sensor data and uncritical sensor configuration data are all transferred through a simple USB cable. The very low power consumption, including automatic entering into sleep mode, allows to use the device not only in fixed installations, but also in mobile applications.

## Reference Documents

Sensor Datasheet:
http://www.avagotech.com/products/product-detail.jsp?navId=H0,C1,C4992,P94665

Programming Guide to the Oak Sensor Family
(This document is still under construction)

# Hardware Specifications

## Sensor: Avago ADJD-S313-QR999

The ADJD-S313-QR999 is a cost effective, CMOS digital output RGB color sensor in miniature surface-mount package. The IC comes with integrated RGB filters, an analog-to-digital converter and a digital core for communication and sensitivity control. The output allows direct interface to micro-controller or other logic control for further signal processing without the need of any additional components.

## Measurement Range

Color:	Red	0 – 191
	Green	0 – 191
	Blue	0 - 191

For more details, please refer to the sensor datasheet (link in chapter 1.1)

## Supported Sensor Features

Read RGB value

Photodiode Size (configurable)

Integration time (configurable)

Sample rate adjustable

## USB Interface

Interface:	USB 2.0 Full Speed (12Mbit/s)

Connector:	Standard USB Mini-B

Device Class:	HID

Sampling Rate: 10ms to 65s, user adjustable

Report Rate:	1ms to 65s, user adjustable

# Software Specifications

All Oak Sensors are implemented as HID devices. Thus driver support is built into all major operating systems.

Captured sensor Data is transmitted through an INTERRUPT IN reports. Therefore real time processing can be guaranteed. This data can be received by the host using regular file read operations. Chapter  describes the contents of this report.

On an independent communication channel, sensor configuration is done using FEATURE reports that are 32 Bytes in length. Special operating system calls exist to transmit / receive feature reports. Chapter  shows the structure of a feature report for each supported command.

Please refer also to the document “Programming Guide to the Oak Sensor Family“ for more details.

## INTERRUPT IN Report Contents (Real time data)

16 Bit	Frame Number	10-3	s

16 Bit	Red		[no unit],

16 Bit	Green		[no unit],

16 Bit	Blue		[no unit],

## FEATURE Report Commands

### Report Mode

| Byte# | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Content | GnS | Tgt | 0x01 | 0x00 | 0x00 | RPTMODE |

GnS:	0 = Set
1 = Get

Tgt	0 = RAM
1 = Flash

RPTMODE:	0 = After Sampling  (Factory Default)
1 = After Change
2 = Fixed Rate

### LED Mode

| Byte# | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Content | GnS | Tgt | 0x01 | 0x01 | 0x00 | LEDMODE |

GnS:	0 = Set
1 = Get

Tgt	0 = RAM
1 = Flash

LEDMODE:	0 = Off  (Factory Default)
1 = On
2 = Blink Slowly
3 = Blink Fast
4 = Blink 4 pulses

### Report Rate

Number of milliseconds between two IN reports. This parameter will only be regarded if Report Mode = 2 (fixed rate)

| Byte# | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Content | GnS | Tgt | 0x02 | 0x00 | 0x00 | RptRate LSB | RptRate MSB |

GnS:	0 = Set
1 = Get

Tgt	0 = RAM
1 = Flash

RptRate:	Report Rate [ms]

### Sample Rate

This is the actual sample rate the sensor is working on.  If Report Mode = 0 (After Sampling) this is also the rate at which the device reports values to the host PC.

| Byte# | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Content | GnS | Tgt | 0x02 | 0x01 | 0x00 | SampRate LSB | SampRate MSB |

GnS:	0 = Set
	1 = Get

Tgt	0 = RAM
	1 = Flash

SampRate:	Sample Rate [ms]

### User Device Name

| Byte# | 0 | 1 | 2 | 3 | 4 | 5-25 |
| --- | --- | --- | --- | --- | --- | --- |
| Content | GnS | Tgt | 0x15 | 0x00 | 0x00 | UsrDevName |

GnS:	0 = Set
1 = Get

Tgt	0 = RAM
1 = Flash

UsrDevName:	User defined name for the whole device 
Null-terminated string, max. 20+1 characters

### User Channel Name

| Byte# | 0 | 1 | 2 | 3 | 4 | 5-25 |
| --- | --- | --- | --- | --- | --- | --- |
| Content | GnS | Tgt | 0x15 | ChP1 | 0x00 | UsrChName |

GnS:	0 = Set
1 = Get

Tgt	0 = RAM
1 = Flash

ChP1	1 = Channel 0 (Frame Number)
2 = Channel 1 (Illuminance)

UsrChName:	User defined name for the channel 
Null-terminated string, max. 20+1 characters

### Photodiode Size

| Byte# | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Content | GnS | 0x03 | 0x01 | ADDR | 0x00 | PDAS |

GnS:	0 = Set
1 = Get

ADDR:	0x0B = PDASR
0x0A = PDASG
0x09 = PDASB

PDAS	0x01 = ¼
0x03 = ½
0x07 = ¾	(reset value)
0x0F = Full

For more details refer to the sensor datasheet, linked in chapter 1.1.

### Integration Time

| Byte# | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Content | GnS | 0x03 | 0x01 | ADDR | 0x00 | TINT |

GnS:	0 = Set
1 = Get

ADDR:	0x11 = TINTR
0x10 = TINTG
0x0F = TINTB

TINT	0x00 = Integration Time Slot = 1
0x01 = Integration Time Slot = 2
…

0x07 = Integration Time Slot = 8	(reset value)
…
0x0E = Integration Time Slot = 15
0x0F = Integration Time Slot = 16

For more details refer to the sensor datasheet, linked in chapter 1.1.

# Technical Specifications

## Electrical Specifications

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Iq1) | Operating current |  |  |  | 20 | mA |
| IStby | Standby current | No USB activity |  |  | 500 | A |

1) The maximum operating current is mainly influenced by the on board LED.

## Mechanical Dimensions

The PCB is designed to be mounted using two standard M2 screws. There are no components on the back side of the pcb.

Figure  1:  Mechanical dimensions of the Oak RGB sensor (Assembly drawing shows a different sensor)

## RoHS Compliance

Unless otherwise stated, all Toradex products comply with the European Union’s Directive 2002/95/EC: "Restrictions of Hazardous Substances".

#### Revision history Date; Doc. Rev.; Changes. 26-Jun-2007; Rev. 0.9; Preliminary Release. #### Disclaimer: Toradex AG reserves the right to make changes, without notice, to any product, including circuits and/or software described or contained in this datasheet. Toradex AG assumes no responsibility or liability for the use of the described product(s), conveys no license or title under any patent, copyright, or mask work rights to these products, and makes no representations or warranties that these products are free from patent, copyright, or mask work right infringement, unless otherwise specified. #### Trademark Acknowledgement: Brand and product names are trademarks or registered trademarks of their respective owners.